Submitted By:

Noor-Ul-Ain – 2025-CE-83

Assignment:

Lab No. 7

Submitted To:

Professor Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

# Experiment

This experiment studies how an operational amplifier (IC 741) amplifies input signals in two modes:

- Inverting Amplifier → Output is amplified but 180° out of phase

- Non-Inverting Amplifier → Output is amplified and in phase

The relationship between input and output voltage depends on resistor values (gain).

# Steps / Procedure

## Inverting Amplifier

- Connect the circuit on breadboard using IC 741.

- Apply AC input signal to pin 2 (inverting terminal) through resistor.

- Ground pin 3 (non-inverting terminal).

- Connect feedback resistor between output (pin 6) and pin 2.

- Provide ±VCC supply (pin 7 = +V, pin 4 = –V).

- Connect oscilloscope:

  - CH A → Input

  - CH B → Output

- Observe waveform → output is inverted.

### connections:

- Input signal → Pin 2 (inverting input) via resistor (R1)

- Feedback resistor (Rf) → between Pin 6 (output) and Pin 2

- Pin 3 (non-inverting) → Ground

### Formula (important for viva):

Av=−Rf / Rin

### Example:

- R1 = 1kΩ

- Rf = 2kΩ
👉 Gain = -2

## Non-Inverting Amplifier

- Connect circuit using IC 741.

- Apply input signal to pin 3 (non-inverting terminal).

- Connect resistor network between pin 2 and output.

- Provide dual power supply (±VCC).

- Connect oscilloscope same way.

- Observe waveform → output is same phase.

### Connections:

- Input → Pin 3 (non-inverting input)

- Pin 2 → connected to:

  - Ground via R1

  - Output via Rf

### Formula:

Av=1+Rf /R1

### Example:

- R1 = 1kΩ

- Rf = 2kΩ
👉 Gain = 3