CIRCUIT ANALYSIS LAB REPORT

LAB # 4

## Title

Measurement of Voltage and Current in Parallel Circuits using Digital Multimeter in Proteus Design Suite

## Objective

The objective of this lab was to measure current through resistors and voltage across specific nodes in parallel circuits using a Digital Multimeter (DMM) and to verify theoretical concepts through simulation in Proteus Design Suite.

## Apparatus / Software Used

- Proteus Design Suite

- Digital Multimeter (DMM)

- Resistors

- DC Power Supply

## Theory

In electrical circuits, voltage, current, and resistance are fundamental quantities.

- Voltage is measured across two points in a circuit

- Current is measured by placing the ammeter in series with the branch

- Resistance controls the flow of current

According to Ohm’s Law, current depends on both applied voltage and resistance.

In a parallel circuit:

- Voltage across all branches is the same

- Current divides among branches depending on resistance

- Lower resistance branches carry higher current

# Task 1: Measurement of Current and Voltage in a Parallel Circuit

## Given Data

- R_1 = 10k

- R_2 = 2k

- R_3 = 1k

- Supply Voltage = 9V

## Procedure

- The circuit was constructed in Proteus Design Suite with three resistors connected in parallel across a 9V source.

- A digital multimeter was used to measure current through each resistor by placing it in series with each branch.

- Voltage was measured between node 2 and node 7 using a voltmeter.

- The simulation was run and readings were observed.

## Observations

- Voltage across each branch remained the same.

- Current varied in each branch depending on resistance.

- The branch with the smallest resistance showed the highest current.

- The branch with the largest resistance showed the lowest current.

## Explanation

In a parallel circuit, all resistors are connected across the same voltage source, so each branch experiences the same voltage.

However, current divides among the branches based on resistance. Lower resistance allows more current to flow, while higher resistance restricts current.

The simulation in Proteus Design Suite confirmed that:

- Voltage remains constant across all branches

- Current distribution depends on resistance

- Total current is shared among different paths

These results agree with the principles of Ohm’s Law.

# Task 2: Measurement of Current and Voltage in Multi-Branch Parallel Circuit

## Given Data

- R_1 = 100ohm

- R_2 = 3k

- R_3 = 5

- R_4 = 10k

- Supply Voltage = 9V

## Procedure

- The circuit was designed in Proteus Design Suite with four resistors connected in parallel between nodes a and e.

- A 9V DC source was applied across the circuit.

- Current through each resistor was measured by inserting the ammeter in each branch.

- Voltage between node a and node e was measured using a voltmeter.

- The simulation was executed and readings were recorded.

## Observations

- Voltage between node a and node e remained constant.

- Current was different in each branch.

- The smallest resistance branch had the highest current.

- The largest resistance branch had the lowest current.

## Explanation

In this parallel network, all resistors are connected between the same two nodes (a and e), so the voltage across each branch is identical.

The current splits into multiple paths, and each branch carries a portion of the total current depending on its resistance value.

The simulation results from Proteus Design Suite verified that:

- Voltage across parallel branches is equal

- Current division follows resistance values

- Multiple current paths exist in parallel circuits

These observations confirm the expected behavior based on Ohm’s Law.

## Conclusion

The lab successfully demonstrated the measurement of voltage and current in parallel circuits using a DMM. The results obtained from Proteus Design Suite matched theoretical expectations and confirmed the fundamental properties of parallel circuits.